Port VLAN 对带宽性能影响测试报告

用例编号：TC-VLAN-PERF-001
测试时间：2026-07-22
测试脚本：ps3/test/esat/nic100/scripts/multihost_port_vlan_performance_tc001.py

# 1. 测试目的

对比在两台直连的 ConnectX-8 网卡上，开启 Port VLAN（VLAN 100）前后：

iperf3 TCP 带宽差异

DPDK testpmd PPS/BPS 差异

包长遍历 64 / 128 / 256 / 512 / 1024 字节，并验证 VLAN tag 真实出现在链路上。

# 2. 测试方案

## 2.1 测试拓扑

Server 与 Client 的 ens9f0np0 物理口直连（或经交换机透传 VLAN 100）。

Server: 10.17.44.71:728，PCI 0000:38:00.0，ConnectX-8，mlx5_core

Client: 10.17.44.73:3322，PCI 0000:38:00.0，ConnectX-8，mlx5_core

## 2.2 测试参数

| 参数 | 取值 |
| --- | --- |
| VLAN ID | 100 |
| 测试包长 | 64, 128, 256, 512, 1024 |
| iperf3 持续时间 | 30 秒 |
| testpmd 运行时间 | 约 20 秒 |
| DPDK 运行模式 | mlx5_core + verbs |

## 2.3 测试工具

iperf3、tcpdump、dpdk-testpmd、ip、ethtool、tmux

# 3. 测试环境

| 角色 | IP:Port | 用户 | 网口 | PCI | 网卡 | 驱动 |
| --- | --- | --- | --- | --- | --- | --- |
| Server | 10.17.44.71:728 | root | ens9f0np0 | 0000:38:00.0 | ConnectX-8 | mlx5_core |
| Client | 10.17.44.73:3322 | root | ens9f0np0 | 0000:38:00.0 | ConnectX-8 | mlx5_core |

注：本环境 dpdk-testpmd 使用 rdma-core/libmlx5 的 verbs 路径，必须保持 mlx5_core 绑定并加载 mlx5_ib。

# 4. 测试步骤与命令

## 4.1 无 VLAN iperf3 基准

Server 侧：

ip addr add 192.168.240.1/24 dev ens9f0np0
ip link set ens9f0np0 up
iperf3 -s -p 5201

Client 侧：

ip addr add 192.168.240.2/24 dev ens9f0np0
ip link set ens9f0np0 up
iperf3 -c 192.168.240.1 -p 5201 -t 30 -B 192.168.240.2 -l <pkt_len> -f m

## 4.2 Port VLAN iperf3

Server 侧：

ip link add link ens9f0np0 name ens9f0np0.100 type vlan id 100
ip addr add 192.168.100.1/24 dev ens9f0np0.100
ip link set ens9f0np0.100 up
iperf3 -s -p 5201

Client 侧：

ip link add link ens9f0np0 name ens9f0np0.100 type vlan id 100
ip addr add 192.168.100.2/24 dev ens9f0np0.100
ip link set ens9f0np0.100 up
iperf3 -c 192.168.100.1 -p 5201 -t 30 -B 192.168.100.2 -l <pkt_len> -f m

## 4.3 无 VLAN DPDK testpmd

Server 侧：

modprobe mlx5_ib
echo 1024 > /sys/kernel/mm/hugepages/hugepages-2048kB/nr_hugepages
printf "vlan set filter off 0\n" > /tmp/port_vlan_perf_tc001_s_cmd.txt
timeout 28 dpdk-testpmd -l 0,1 -n 4 -a 0000:38:00.0 -- --forward-mode=rxonly --portmask=0x1 --nb-cores=1 --cmdline-file=/tmp/port_vlan_perf_tc001_s_cmd.txt --auto-start --stats-period=5

Client 侧：

timeout 23 dpdk-testpmd -l 0,1 -n 4 -a 0000:38:00.0 -- --forward-mode=txonly --portmask=0x1 --nb-cores=1 --txpkts=<pkt_len> --auto-start --stats-period=5

## 4.4 Port VLAN DPDK testpmd

Server 侧与 4.3 相同。Client 侧在启动前写入 VLAN insert 命令：

printf "tx_vlan set 0 100\n" > /tmp/port_vlan_perf_tc001_c_cmd.txt
timeout 23 dpdk-testpmd -l 0,1 -n 4 -a 0000:38:00.0 -- --forward-mode=txonly --portmask=0x1 --nb-cores=1 --txpkts=<pkt_len> --cmdline-file=/tmp/port_vlan_perf_tc001_c_cmd.txt --auto-start --stats-period=5

## 4.5 VLAN 生效验证

Server 物理口抓包：

nohup tcpdump -i ens9f0np0 -nn -e -s 0 -w /tmp/port_vlan_verify.pcap vlan 100 >/tmp/tcpdump_verify.log 2>&1 &
iperf3 -s -p 5201

Client VLAN 子接口发流：

iperf3 -c 192.168.100.1 -p 5201 -t 10 -B 192.168.100.2 -l 256 -f m

读取 pcap：

tcpdump -nn -e -r /tmp/port_vlan_verify.pcap | head -30

# 5. 测试结果

## 5.1 iperf3 带宽对比

| 包长 (B) | 无 VLAN (Mbps) | Port VLAN (Mbps) | 差异 |
| --- | --- | --- | --- |
| 64 | 530 | 533 | +0.57% |
| 128 | 1040 | 1030 | -0.96% |
| 256 | 1990 | 2000 | +0.50% |
| 512 | 3830 | 3830 | 0.00% |
| 1024 | 7220 | 7210 | -0.14% |

结论：开启 Port VLAN 后，iperf3 带宽波动在 ±1% 以内，VLAN tag 对 TCP 吞吐无显著影响。

## 5.2 DPDK testpmd 吞吐对比

| 包长 (B) | 模式 | Rx-pps | Rx-bps | 估算 Mbps |
| --- | --- | --- | --- | --- |
| 64 | 无 VLAN | 39,392,895 | 20,169,162,376 | 20,169 |
| 64 | Port VLAN | 39,364,295 | 20,154,519,480 | 20,155 |
| 128 | 无 VLAN | 37,960,135 | 38,871,178,288 | 38,871 |
| 128 | Port VLAN | 37,959,632 | 38,870,663,880 | 38,871 |
| 256 | 无 VLAN | 37,166,322 | 76,116,628,832 | 76,117 |
| 256 | Port VLAN | 37,037,646 | 75,853,099,488 | 75,853 |
| 512 | 无 VLAN | 34,168,581 | 139,954,511,832 | 139,955 |
| 512 | Port VLAN | 34,141,826 | 139,844,921,216 | 139,845 |
| 1024 | 无 VLAN | 32,610,882 | 267,148,347,872 | 267,148 |
| 1024 | Port VLAN | 32,671,210 | 267,642,558,856 | 267,643 |

结论：小包受单核转发限制（约 39~40 Mpps），大包带宽近似线性上升；VLAN 场景与无 VLAN 场景差异均 < 0.35%。

# 6. 测试日志

## 6.1 iperf3 日志摘要

模式：baseline

64B server summary: [  5]   0.00-30.00  sec  1.85 GBytes   530 Mbits/sec                  receiver

128B server summary: [  5]   0.00-30.00  sec  3.63 GBytes  1.04 Gbits/sec                  receiver

256B server summary: [  5]   0.00-30.00  sec  6.94 GBytes  1.99 Gbits/sec                  receiver

512B server summary: [  5]   0.00-30.00  sec  13.4 GBytes  3.83 Gbits/sec                  receiver

1024B server summary: [  5]   0.00-30.00  sec  25.2 GBytes  7.22 Gbits/sec                  receiver

模式：vlan

64B server summary: [  5]   0.00-30.00  sec  1.86 GBytes   533 Mbits/sec                  receiver

128B server summary: [  5]   0.00-30.00  sec  3.58 GBytes  1.03 Gbits/sec                  receiver

256B server summary: [  5]   0.00-30.00  sec  7.00 GBytes  2.00 Gbits/sec                  receiver

512B server summary: [  5]   0.00-30.00  sec  13.4 GBytes  3.83 Gbits/sec                  receiver

1024B server summary: [  5]   0.00-30.00  sec  25.2 GBytes  7.21 Gbits/sec                  receiver

## 6.2 DPDK testpmd 日志摘要

模式：无 VLAN

64B server throughput:

Throughput (since last show)
  Rx-pps:     39392895          Rx-bps:  20169162376
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

128B server throughput:

Throughput (since last show)
  Rx-pps:     37960135          Rx-bps:  38871178288
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

256B server throughput:

Throughput (since last show)
  Rx-pps:     37166322          Rx-bps:  76116628832
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

512B server throughput:

Throughput (since last show)
  Rx-pps:     34168581          Rx-bps: 139954511832
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

1024B server throughput:

Throughput (since last show)
  Rx-pps:     32610882          Rx-bps: 267148347872
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

模式：Port VLAN

64B server throughput:

Throughput (since last show)
  Rx-pps:     39364295          Rx-bps:  20154519480
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

128B server throughput:

Throughput (since last show)
  Rx-pps:     37959632          Rx-bps:  38870663880
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

256B server throughput:

Throughput (since last show)
  Rx-pps:     37037646          Rx-bps:  75853099488
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

512B server throughput:

Throughput (since last show)
  Rx-pps:     34141826          Rx-bps: 139844921216
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

1024B server throughput:

Throughput (since last show)
  Rx-pps:     32671210          Rx-bps: 267642558856
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

## 6.3 VLAN 抓包验证

tcpdump 读取 pcap 前 15 行（已确认存在 802.1Q VLAN 100 tag）：

23:20:27.111979 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.41789 > 192.168.100.1.5201: Flags [.], ack 1, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 0
23:20:27.112018 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 107: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.41789 > 192.168.100.1.5201: Flags [P.], seq 1:38, ack 1, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 37
23:20:27.112024 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.41789: Flags [.], ack 38, win 509, options [nop,nop,TS val 331558731 ecr 2499743305], length 0
23:20:27.112150 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 71: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.56351: Flags [P.], seq 3:4, ack 139, win 509, options [nop,nop,TS val 331558731 ecr 2499743305], length 1
23:20:27.112160 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.56351 > 192.168.100.1.5201: Flags [.], ack 4, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 0
23:20:27.112164 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 71: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.56351: Flags [P.], seq 4:5, ack 139, win 509, options [nop,nop,TS val 331558731 ecr 2499743305], length 1
23:20:27.112211 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.56351 > 192.168.100.1.5201: Flags [.], ack 5, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 0
23:20:27.112215 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 326: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.41789 > 192.168.100.1.5201: Flags [P.], seq 38:294, ack 1, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 256
23:20:27.112217 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.41789: Flags [.], ack 294, win 507, options [nop,nop,TS val 331558731 ecr 2499743305], length 0
23:20:27.112288 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 3142: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.41789 > 192.168.100.1.5201: Flags [P.], seq 294:3366, ack 1, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 3072
23:20:27.112288 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 5862: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.41789 > 192.168.100.1.5201: Flags [.], seq 3366:9158, ack 1, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 5792
23:20:27.112294 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.41789: Flags [.], ack 3366, win 494, options [nop,nop,TS val 331558731 ecr 2499743305], length 0
23:20:27.112297 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.41789: Flags [.], ack 9158, win 464, options [nop,nop,TS val 331558731 ecr 2499743305], length 0
23:20:27.112308 8c:91:3a:4d:2a:54 > 8c:91:3a:4d:42:34, ethertype 802.1Q (0x8100), length 8758: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.41789 > 192.168.100.1.5201: Flags [P.], seq 9158:17846, ack 1, win 502, options [nop,nop,TS val 2499743305 ecr 331558731], length 8688
23:20:27.112316 8c:91:3a:4d:42:34 > 8c:91:3a:4d:2a:54, ethertype 802.1Q (0x8100), length 70: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5201 > 192.168.100.2.41789: Flags [.], ack 17846, win 397, options [nop,nop,TS val 331558731 ecr 2499743305], length 0

# 7. 结论

1. VLAN 生效性：通过 server 物理口 tcpdump 确认，Port VLAN 100 的 tag 真实出现在链路上。

2. iperf3 性能影响：开启 Port VLAN 后，64/128/256/512/1024 字节包长下带宽波动均在 ±1% 以内，无显著性能下降。

3. DPDK testpmd 性能影响：开启 VLAN insert 和关闭 VLAN filter 后，各包长 PPS/BPS 差异 < 0.35%，VLAN 处理对硬件转发性能影响可忽略。

4. 总体结论：在本测试环境（ConnectX-8、mlx5_core、直连）下，Port VLAN 对 iperf3/testpmd 带宽性能影响极小。

# 8. 附录：原始日志清单

所有原始日志已保存到 docs/port_vlan_perf_report/：

iperf_s_<mode>_<len>.log / iperf_c_<mode>_<len>.log

testpmd_s_<len>_<mode>.log / testpmd_c_<len>_<mode>.log

vlan_verify_tcpdump_sample.log