Port VLAN（SR-IOV VF VLAN）对带宽性能影响测试报告

用例编号：TC-VLAN-PERF-001-VF
测试时间：2026-07-22
测试脚本：ps3/test/esat/nic100/scripts/multihost_port_vlan_performance_tc001_vf.py

# 1. 测试目的

使用 SR-IOV VF Port VLAN 方式配置 VLAN 100，对比开启前后：

iperf3 TCP 带宽差异

DPDK testpmd PPS/BPS 差异

包长遍历 64 / 128 / 256 / 512 / 1024 字节，并验证 VLAN tag 真实出现在链路上。

# 2. 测试方案

## 2.1 测试拓扑

Server 与 Client 的 ens9f0np0 物理口直连，每端创建 VF 0（PCI 0000:38:00.2，接口 ens9f0v0）。

Server: 10.17.44.71:728，PF/VF PCI 0000:38:00.0 / 0000:38:00.2，ConnectX-8，mlx5_core

Client: 10.17.44.73:3322，PF/VF PCI 0000:38:00.0 / 0000:38:00.2，ConnectX-8，mlx5_core

## 2.2 测试参数

| 参数 | 取值 |
| --- | --- |
| VLAN ID | 100 |
| 测试包长 | 64, 128, 256, 512, 1024 |
| iperf3 持续时间 | 30 秒 |
| testpmd 运行时间 | 约 10 秒 |
| DPDK 运行模式 | VF PCI + mlx5_core + verbs |
| Port VLAN 命令 | ip link set ens9f0np0 vf 0 vlan 100 |

## 2.3 测试工具

iperf3、tcpdump、dpdk-testpmd、ip、ethtool、tmux

# 3. 测试环境

| 角色 | IP:Port | 用户 | PF 网口 | VF 网口 | PF/VF PCI | 网卡 | 驱动 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Server | 10.17.44.71:728 | root | ens9f0np0 | ens9f0v0 | 0000:38:00.0 / 0000:38:00.2 | ConnectX-8 | mlx5_core |
| Client | 10.17.44.73:3322 | root | ens9f0np0 | ens9f0v0 | 0000:38:00.0 / 0000:38:00.2 | ConnectX-8 | mlx5_core |

注：本环境 dpdk-testpmd 使用 rdma-core/libmlx5 的 verbs 路径，必须保持 mlx5_core 绑定并加载 mlx5_ib。

# 4. 测试步骤与命令

## 4.1 环境准备

echo 1 > /sys/bus/pci/devices/0000:38:00.0/sriov_numvfs
modprobe mlx5_ib
echo 1024 > /sys/kernel/mm/hugepages/hugepages-2048kB/nr_hugepages

## 4.2 无 VLAN iperf3 基准

Server 侧：

ip link set ens9f0np0 vf 0 vlan 0
ip addr add 192.168.240.1/24 dev ens9f0v0
ip link set ens9f0v0 up
iperf3 -s -p 5201

Client 侧：

ip link set ens9f0np0 vf 0 vlan 0
ip addr add 192.168.240.2/24 dev ens9f0v0
ip link set ens9f0v0 up
iperf3 -c 192.168.240.1 -p 5201 -t 30 -B 192.168.240.2 -l <pkt_len> -f m

## 4.3 Port VLAN iperf3

Server 侧：

ip link set ens9f0np0 vf 0 vlan 100
ip addr add 192.168.100.1/24 dev ens9f0v0
ip link set ens9f0v0 up
iperf3 -s -p 5201

Client 侧：

ip link set ens9f0np0 vf 0 vlan 100
ip addr add 192.168.100.2/24 dev ens9f0v0
ip link set ens9f0v0 up
iperf3 -c 192.168.100.1 -p 5201 -t 30 -B 192.168.100.2 -l <pkt_len> -f m

## 4.4 无 VLAN DPDK testpmd

Server 侧：

printf 'vlan set filter off 0\n' > /tmp/vf_tpmd_s_cmd.txt
timeout 20 dpdk-testpmd -l 0,1 -n 4 -a 0000:38:00.2 -- --forward-mode=rxonly --portmask=0x1 --nb-cores=1 --cmdline-file=/tmp/vf_tpmd_s_cmd.txt --auto-start --stats-period=3

Client 侧（广播目的 MAC）：

timeout 15 dpdk-testpmd -l 0,1 -n 4 -a 0000:38:00.2 -- --forward-mode=txonly --portmask=0x1 --nb-cores=1 --txpkts=<pkt_len> --eth-peer=0,ff:ff:ff:ff:ff:ff --auto-start --stats-period=3

## 4.5 Port VLAN DPDK testpmd

先执行 `ip link set ens9f0np0 vf 0 vlan 100`，其余命令与 4.4 相同。

## 4.6 VLAN 生效验证

Server PF 物理口抓包：

nohup tcpdump -i ens9f0np0 -nn -e -s 0 -w /tmp/vf_verify.pcap >/tmp/tcpdump_vf_verify.log 2>&1 &
iperf3 -s -p 5201

Client VF 发流：

iperf3 -c 192.168.100.1 -p 5201 -t 10 -B 192.168.100.2 -l 256 -f m

读取 pcap：

tcpdump -nn -e -r /tmp/vf_verify.pcap | head -30

# 5. 测试结果

## 5.1 iperf3 带宽对比

| 包长 (B) | 无 VLAN (Mbps) | Port VLAN (Mbps) | 差异 |
| --- | --- | --- | --- |
| 64 | 535.0 | 537.0 | +0.37% |
| 128 | 1030.0 | 1040.0 | +0.97% |
| 256 | 2000.0 | 2010.0 | +0.50% |
| 512 | 3860.0 | 3780.0 | -2.07% |
| 1024 | 7130.0 | 6990.0 | -1.96% |

结论：开启 SR-IOV VF Port VLAN 后，iperf3 带宽波动在 ±2% 以内，VLAN tag 对 TCP 吞吐无显著影响。

## 5.2 DPDK testpmd 吞吐对比

| 包长 (B) | 模式 | Rx-pps | Rx-bps | 估算 Mbps |
| --- | --- | --- | --- | --- |
| 64 | 无 VLAN | 36,095,870 | 18,481,085,816 | 18,481 |
| 64 | Port VLAN | 39,324,793 | 20,134,294,488 | 20,134 |
| 128 | 无 VLAN | 35,543,469 | 36,396,512,888 | 36,396 |
| 128 | Port VLAN | 38,663,311 | 39,591,231,032 | 39,591 |
| 256 | 无 VLAN | 34,520,369 | 70,697,717,512 | 70,697 |
| 256 | Port VLAN | 37,371,757 | 76,537,360,368 | 76,537 |
| 512 | 无 VLAN | 32,260,786 | 132,140,181,072 | 132,140 |
| 512 | Port VLAN | 34,809,144 | 142,578,255,232 | 142,578 |
| 1024 | 无 VLAN | 30,189,528 | 247,312,620,464 | 247,312 |
| 1024 | Port VLAN | 30,864,956 | 252,845,720,784 | 252,845 |

结论：VF Port VLAN 场景下各包长 PPS/BPS 与无 VLAN 场景基本持平甚至略高，VLAN 处理对硬件转发性能影响可忽略。

# 6. 测试日志

## 6.1 iperf3 日志摘要

模式：baseline

64B server summary: [  5]   0.00-30.00  sec  1.87 GBytes   535 Mbits/sec                  receiver

128B server summary: [  5]   0.00-30.00  sec  3.59 GBytes  1.03 Gbits/sec                  receiver

256B server summary: [  5]   0.00-30.00  sec  7.00 GBytes  2.00 Gbits/sec                  receiver

512B server summary: [  5]   0.00-30.00  sec  13.5 GBytes  3.86 Gbits/sec                  receiver

1024B server summary: [  5]   0.00-30.00  sec  24.9 GBytes  7.13 Gbits/sec                  receiver

模式：vlan

64B server summary: [  5]   0.00-30.00  sec  1.87 GBytes   537 Mbits/sec                  receiver

128B server summary: [  5]   0.00-30.00  sec  3.63 GBytes  1.04 Gbits/sec                  receiver

256B server summary: [  5]   0.00-30.00  sec  7.03 GBytes  2.01 Gbits/sec                  receiver

512B server summary: [  5]   0.00-30.00  sec  13.2 GBytes  3.78 Gbits/sec                  receiver

1024B server summary: [  5]   0.00-30.00  sec  24.4 GBytes  6.99 Gbits/sec                  receiver

## 6.2 DPDK testpmd 日志摘要

模式：无 VLAN

64B server throughput:

Throughput (since last show)
  Rx-pps:     36095870          Rx-bps:  18481085816
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

128B server throughput:

Throughput (since last show)
  Rx-pps:     35543469          Rx-bps:  36396512888
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

256B server throughput:

Throughput (since last show)
  Rx-pps:     34520369          Rx-bps:  70697717512
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

512B server throughput:

Throughput (since last show)
  Rx-pps:     32260786          Rx-bps: 132140181072
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

1024B server throughput:

Throughput (since last show)
  Rx-pps:     30189528          Rx-bps: 247312620464
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

模式：Port VLAN

64B server throughput:

Throughput (since last show)
  Rx-pps:     39324793          Rx-bps:  20134294488
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

128B server throughput:

Throughput (since last show)
  Rx-pps:     38663311          Rx-bps:  39591231032
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

256B server throughput:

Throughput (since last show)
  Rx-pps:     37371757          Rx-bps:  76537360368
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

512B server throughput:

Throughput (since last show)
  Rx-pps:     34809144          Rx-bps: 142578255232
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

1024B server throughput:

Throughput (since last show)
  Rx-pps:     30864956          Rx-bps: 252845720784
  Tx-pps:            0          Tx-bps:            0
  ############################################################################
Telling cores to stop...
Waiting for lcores to finish...

## 6.3 VLAN 抓包验证

tcpdump 读取 pcap 样本（已确认存在 802.1Q VLAN 100 tag）：

02:01:13.697805 b6:5c:03:91:df:9e > 01:00:5e:00:00:fb, ethertype 802.1Q (0x8100), length 127: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.1.5353 > 224.0.0.251.5353: 0*- [0q] 2/0/0 (Cache flush) PTR linux-8.local., (Cache flush) A 192.168.100.1 (81)
02:01:14.018172 f2:a6:ff:47:5d:e2 > 01:00:5e:00:00:fb, ethertype 802.1Q (0x8100), length 129: vlan 100, p 0, ethertype IPv4 (0x0800), 192.168.100.2.5353 > 224.0.0.251.5353: 0*- [0q] 2/0/0 (Cache flush) PTR fpga44-73.local., (Cache flush) A 192.168.100.2 (83)
02:01:16.803857 f2:a6:ff:47:5d:e2 > ff:ff:ff:ff:ff:ff, ethertype 802.1Q (0x8100), length 60: vlan 100, p 0, ethertype ARP (0x0806), Request who-has 192.168.100.1 tell 192.168.100.2, length 42

# 7. 结论

1. VLAN 生效性：通过 `ip link set ens9f0np0 vf 0 vlan 100` 配置后，server PF 物理口 tcpdump 确认链路上存在 802.1Q VLAN 100 tag。

2. iperf3 性能影响：开启 VF Port VLAN 后，64/128/256/512/1024 字节包长下带宽波动在 ±2% 以内，无显著性能下降。

3. DPDK testpmd 性能影响：VF Port VLAN 场景下各包长 PPS/BPS 与无 VLAN 场景基本持平甚至略高，VLAN 处理对硬件转发性能影响可忽略。

4. 总体结论：在本测试环境（ConnectX-8、mlx5_core、VF、直连）下，使用 SR-IOV VF Port VLAN 对 iperf3/testpmd 带宽性能影响极小。

# 8. 附录：原始日志清单

所有原始日志已保存到 docs/port_vlan_perf_report_vf/：

iperf_s_<mode>_<len>.log / iperf_c_<mode>_<len>.log

testpmd_s_<len>_<mode>.log / testpmd_c_<len>_<mode>.log

vlan_verify_tcpdump_sample.log

summary.json

# 9. 注意事项

1. VF-to-VF 单播：本环境硬件 e-switch FDB 无法学习到对端 VF MAC，导致单播报文无法到达。testpmd 使用广播目的 MAC 进行测试。

2. VF MAC：`ip link set <pf> vf <id> mac <mac>` 设置的是管理员 MAC，实际 ens9f0v0 接口可能显示不同 MAC。

3. DPDK verbs 模式：必须保持 mlx5_core 绑定并加载 mlx5_ib。